This is one of the most important topics in modern VLSI Physical Design. Since you are learning power planning in Innovus, understanding UPF/CPF will help you understand why we create multiple power nets, power rings, power straps, isolation cells, etc.

I'll explain it from the beginning.

# Why do we need UPF/CPF?

Earlier chips had only one power supply.

Entire Chip

---------------------

VDD = 1.0V

VSS = GND

---------------------

Everything always remained ON.

Power consumption was high.

Today's SoCs (mobile processors, AI chips, automotive chips) contain many blocks.

Example : CPU, GPU, USB, Bluetooth, DDR, Camera, Display

Not every block is required all the time.

Example

You are only listening to music.

Bluetooth  -> ON

CPU         -> ON

GPU         -> OFF

Camera      -> OFF

Video Codec -> OFF

Why keep GPU powered if it is unused?

Answer: We shut off its power. This is called Power Gating

# What is UPF?

UPF = Unified Power Format

It is an IEEE standard.

A UPF file tells the EDA tools

"This block belongs to this power domain."

"This block uses this voltage."

"Insert isolation cells here."

"Insert retention registers here."

"Use these sleep transistors."

Instead of manually inserting these cells into RTL, we describe the power intent in a UPF file.

# CPF CPF = Common Power Format, Created by Cadence.

Purpose is almost identical to UPF.

Nowadays UPF is much more widely used.

# What is Power Intent?

Power intent means : How power should behave in the chip.

This information is stored inside UPF.

# Main Concepts in UPF

Let's go one by one.
1. Power Domain

A power domain is A group of cells that share the same ON/OFF behavior.

Example

-----------------------

Chip

-----------------------

CPU , GPU, Bluetooth and Camera

Suppose

CPU should never shut down.

GPU should sleep.

Bluetooth should sleep.

Camera should sleep.

Then

Power Domain 1 : CPU :Always ON

Power Domain 2 : GPU : Can OFF

Power Domain 3 : Bluetooth : Can OFF

Power Domain 4 : Camera : Can OFF

Each is a different power domain.

# Important Point

Power Domain is about : ON/OFF control , NOT voltage.

# 2. Voltage Domain

Voltage domain means : Cells working at the same voltage.

Example 
CPU : 1.0V
GPU : 0.8V
DDR : 1.2V
Each becomes a different voltage domain.

# Difference

Power Domain: ON/OFF behavior

Voltage Domain : Operating voltage

Example : CPU ; Always ON : 1.0V
         GPU : Sleep Mode : 1.0V

same voltage but Different power domains

Another example

GPU : 0.8V

DSP : 1.0V

Always ON , Different voltage domains and Same power domain possible.

# One voltage domain can contain multiple power domains

# One power domain can also contain multiple voltage domains

Possible in some advanced designs.

# 3. Supply Net

Example

VDD_CPU

VDD_GPU

VDD_IO

VSS

UPF defines these supply nets.

# 4. Supply Set

A pair of power and ground.

Example

Power : VDD_GPU

Ground : VSS

Together Supply Set

# 5. Power Switch

This is a transistor.

# Sleep Transistor

Power switch = Sleep transistor.

Usually a very large transistor.

Placed near the power grid.

VDD

|

Sleep Transistor

|

Power Domain

# Why called Sleep Transistor?

Because it disconnects power during sleep mode.

# 6. Power Gating

Entire concept.

.

# 7. Isolation Cell

Very important interview question.

CPU receives garbage.

Isolation cell solves this.

Without isolation

GPU OFF

Output = X

CPU

Error

With isolation

GPU OFF

Output = 0

CPU continues safely.

# Where is Isolation Cell placed?

# 8. Retention Register

Suppose

GPU has Counter = 250

GPU turns OFF.

Power removed. Counter becomes 0

Information lost.

Retention Register keeps important values.

# 9. Always ON Cells

Some logic must remain ON.

Examples

Power controller

Reset controller

Clock controller

Wake-up logic

These cells belong to the Always-On power domain.

# 10. Level Shifter

Suppose

CPU : 1.2V
GPU :  0.8V

Signal

CPU

↓

GPU

Voltage mismatch.

Need

CPU

↓

Level Shifter

↓

GPU

Two cases

High → Low

Low → High

Different level shifters may be used depending on the direction and library.

# Relation between Isolation and Level Shifter

# Role of UPF During the Design Flow

RTL

↓

Read UPF

↓

Synthesis

The synthesis tool inserts:

- Isolation cells

- Retention cells

- Level shifters

- Power switches (depending on methodology)

Then : Netlist -> Floorplan -> Power Planning -> Placement -> CTS -> Routing

The implementation tool uses the UPF information throughout the flow.

# In Physical Design (Innovus), what do PD engineers do?

Once the synthesized netlist (with UPF intent) reaches Physical Design, you typically:

- Create separate power grids (rings, straps, rails) for each power domain.

- Connect the correct supply nets (for example, VDD_CPU, VDD_GPU, VDD_IO).

- Ensure isolation cells, level shifters, retention cells, and always-on cells are placed in legal locations.

- Route always-on power separately where required.

- Verify power-domain connectivity and power intent.

- Check that power switches and switched power nets are connected correctly.

- Run low-power checks (LP verification) to ensure the implementation matches the UPF intent.
Complete Example

CHIP

-------------------------------------------------

CPU Domain

1.0V

Always ON

|

|

Isolation Cell

|

-------------------

|                 |

GPU Domain       Camera Domain

0.8V             1.0V

Sleep Switch     Sleep Switch

|                 |

Retention Reg      Retention Reg

-------------------------------------------------

Here:

- Power Domain: CPU, GPU, Camera (different ON/OFF behavior)

- Voltage Domain: CPU = 1.0V, GPU = 0.8V, Camera = 1.0V

- Sleep Transistor: Disconnects power to GPU and Camera

- Isolation Cell: Protects the CPU from invalid signals when another domain is OFF

- Retention Register: Saves important state before power is removed

- Level Shifter: Needed between CPU (1.0V) and GPU (0.8V) if signals cross voltage domains

Since you're currently studying power planning in Innovus, the next logical topic is how a UPF file is written (commands such as create_power_domain, create_supply_port, create_supply_net, set_domain_supply_net, create_power_switch, set_isolation, set_retention, and set_level_shifter) and how these commands map to what you see during floorplanning and placement. That connection makes the concepts much easier to understand.