# Complete VLSI STA and Physical Design Notes

## ASIC Flow

Complete ASIC Flow:
1. Specification
2. RTL Design
3. Functional Verification
4. Synthesis
5. DFT (Design for Test)
6. Physical Design
7. STA & Signoff
8. Fabrication
9. Packaging
10. Testing & Validation

ASIC means Application-Specific Integrated Circuit.

## Physical Design (PD) Flow

Goal:
Convert logical netlist into manufacturable chip layout.

## Timing Paths

A timing path is the complete path through which data travels from one point to another point in the chip.

Basic Timing Path:
Launch Flip-Flop → Combinational Logic → Capture Flip-Flop

Main Components:
1. Launch Flip-Flop
2. Combinational Logic
3. Capture Flip-Flop

Types of Timing Paths:
- Register to Register
- Input to Register
- Register to Output
- Input to Output

Critical Path:
The path having maximum delay is called critical timing path.

## 1. Net Delay and Cell Delay

Cell Delay:
Cell delay is the delay produced inside a logic gate or standard cell.
Example: inverter delay, NAND gate delay.

Net Delay:
Net delay is the delay caused by interconnect wires between cells.
It depends on:
- Wire resistance (R)
- Wire capacitance (C)
- Wire length

Total Path Delay Formula:
Total Delay = Cell Delay + Net Delay

Important Points:
- In modern technologies, net delay is often larger than cell delay.
- Placement and routing strongly affect net delay.

## 2. Setup Time and Hold Time

Setup Time:
Minimum time before clock edge during which data must remain stable.

Hold Time:
Minimum time after clock edge during which data must remain stable.

Setup Check Formula:
Clock Period >= Clock-to-Q + Combinational Delay + Setup Time + Skew

Hold Check Formula:
Clock-to-Q + Minimum Delay >= Hold Time + Skew

Violations:
- Large delay causes setup violation.
- Very small delay causes hold violation.

## 3. Clock Skew

Clock skew means difference in arrival time of clock signal between launch and capture flip-flops.

Positive Skew:
Capture clock arrives later.

Negative Skew:
Capture clock arrives earlier.

Causes of Skew:
- Unequal wire lengths
- Buffer delays
- Clock tree imbalance

Skew Formula:
Skew = Capture Clock Arrival - Launch Clock Arrival

## 4. Impact of Clock Skew

Positive Skew:
- Improves setup timing
- Worsens hold timing

Negative Skew:
- Worsens setup timing
- Improves hold timing

Important Understanding:
Setup and hold timing are always affected opposite to each other by skew.

## Timing Arcs

Timing arc means relationship between input transition and output transition inside a standard cell.

Examples:
- Inverter: A → Y
- NAND Gate: A → Y, B → Y

Types of Timing Arcs:
- Combinational Timing Arc
- Sequential Timing Arc

## Unateness

Unateness describes output behavior with respect to input transition.

Types:
1. Positive Unate:
   Input and output move in same direction.
   Example: Buffer

2. Negative Unate:
   Input and output move in opposite direction.
   Example: Inverter

3. Non-Unate:
   Output direction cannot be predicted.
   Example: XOR, MUX
Unateness is important for STA delay calculation and transition propagation.

## 6. False Paths

False path is a logically impossible or unused timing path.

STA ignores these paths because data never propagates in real operation.

Examples:
- MUX mutually exclusive paths
- Debug logic inactive during normal mode

Importance:
Reduces unnecessary timing violations.

## 7. Half Cycle Paths

Half-cycle path uses opposite clock edges.

Example:
- Launch on positive edge
- Capture on negative edge

Available timing becomes half clock period.

Formula:
Available Time = Clock Period / 2

## 8. Multi-Cycle Paths (MCP)

Multi-cycle path is a path allowed to take more than one clock cycle.

Example:
A complex arithmetic block may need 2 or 3 cycles.

MCP Setup Formula:
Required Time = N × Clock Period
Where N = number of cycles.

Importance:
Prevents unnecessary setup violations on slow functional paths.

## 9. Timing Arcs and Unateness

Timing Arc:
Relationship between input pin transition and output pin transition.

Unateness Types:
1. Positive Unate
   Input ↑ → Output ↑
2. Negative Unate
   Input ↑ → Output ↓
3. Non-Unate
   Output transition unpredictable.

Examples:
- Buffer → Positive Unate
- Inverter → Negative Unate
- XOR → Non-Unate

## Important STA Concepts Summary Table

| Concept | Improves | Worsens |
| --- | --- | --- |
| Positive Skew | Setup | Hold |
| Negative Skew | Hold | Setup |
| Large Delay | Hold | Setup |
| Small Delay | Setup | Hold |

## Basic Timing Diagram

The timing diagram above shows clock and data relationship used for setup and hold analysis.