# Static Timing Analysis (STA) – Complete Timing Impact Guide

## 1. What is STA?

Static Timing Analysis (STA) is the process of verifying whether all timing paths in a design meet setup and hold requirements without running simulations.

STA checks:

- Setup Timing

- Hold Timing

- Clock Timing

- Path Delays

- Timing Margins

The final result of STA is Timing Slack.

# 2. Basic Timing Path

A timing path consists of:

Launch Flip-Flop → Combinational Logic → Capture Flip-Flop

Components:

- Launch Clock Delay

- Clock-to-Q Delay

- Combinational Logic Delay

- Interconnect Delay

- Capture Clock Delay

# 3. Arrival Time (DAT)

Data Arrival Time (DAT) is the time required for data to reach the capture flip-flop.

Formula:  DAT = Launch Clock Latency + Clock-to-Q Delay + Cell Delays +Net Delays

Example:

Launch Clock Delay = 0.5 ns

Clock-to-Q = 0.2 ns

Logic Delay = 1.5 ns

Net Delay = 0.8 ns

DAT = 3.0 ns

# 4. Required Time (DRT)

Data Required Time (DRT) is the latest time by which data must reach the capture flip-flop.

Setup Analysis:  DRT = Clock Period + Capture Clock Latency – (Setup Time + Clock Uncertainty )

Example:

Clock Period = 10 ns

Capture Clock Delay = 1 ns

Setup Time = 0.2 ns

Uncertainty = 0.1 ns

DRT = 10 + 1 - 0.2 - 0.1

DRT = 10.7 ns

# 5. Setup Slack

Formula: Setup Slack = Required Time - Arrival Time

Example:

DRT = 10.7 ns

DAT = 10.2 ns

Setup Slack = 0.5 ns

Positive Slack → Pass

Negative Slack → Violation

# 6. Hold Slack

Formula: Hold Slack = Arrival Time - Required Time

Example:

Arrival Time = 0.8 ns , Required Time = 0.5 ns

Hold Slack = 0.3 ns

Positive Slack → Pass

Negative Slack → Violation

# 7. Setup Time

Minimum time before clock edge during which data must remain stable.

Example: Data must arrive 100 ps before the active clock edge.

# 8. Hold Time

Minimum time after clock edge during which data must remain stable.

Example: Data must remain stable 50 ps after the clock edge.

# 9. Clock Latency

Time taken by clock to travel from clock source to flip-flop.

Clock Source
|
Buffers
|
FF

Formula:

Clock Latency = Source Latency + Network Latency

# 10. Clock Skew Difference between launch and capture clock arrival times.

Formula:

Clock Skew = Capture Clock Arrival - Launch Clock Arrival

Example: Launch Clock = 1.0 ns , Capture Clock = 1.2 ns  and Skew = 0.2 ns

Positive Skew: Helps Setup and Hurts Hold.

Negative Skew: Helps Hold and Hurts Setup

# 11. Clock Jitter

Variation in clock edge position.

Ideal: 10 ns , 20 ns and 30 ns.

Actual: 10.05 ns, 19.98 ns , and 30.08 ns

This variation is Jitter.

Jitter reduces timing margin.

# 12. Clock Uncertainty

Represents:

- Jitter

- Clock Variation

- Modeling Margin

Example: set_clock_uncertainty 0.1

Setup Required Time decreases by 100 ps.

Slack becomes worse.

# 13. OCV (On-Chip Variation)

Manufacturing variations cause cells to behave differently.

Example: Nominal Delay = 100 ps

Actual Cell Delays:  Cell A = 95 ps , Cell B = 105 ps and Cell C = 110 ps

STA must account for these variations.

Therefore OCV is applied.

Setup Analysis:  Launch Path → Fast and Capture Path → Slow

Hold Analysis: Launch Path → Slow and Capture Path → Fast

OCV increases pessimism but improves accuracy.

# 14. Derate

Derate modifies delays to model variation.

Example: Cell Delay = 100 ps, Early Derate = 0.95 and Late Derate = 1.05.

Calculations: Early Delay = 95 ps and Late Delay = 105 ps.

Command Example:

set_timing_derate -early 0.95

set_timing_derate -late 1.05

Derates directly affect slack.

# 15. AOCV (Advanced OCV)

Traditional OCV uses the same derate for every path.

Not realistic.

AOCV considers:

- Logic Depth

- Path Length

- Cell Location

Example:

2-stage path = 8% derate

20-stage path = 3% derate

More accurate than OCV.

# 16. POCV (Parametric OCV)

Modern statistical variation method.

Uses: Mean Delay and Sigma Variation.

Example: Mean Delay = 100 ps and Sigma = 5 ps

Tool statistically computes timing variation.

Advantages: More accurate , Less pessimistic and Widely used in advanced nodes

# 17. CPPR (Common Path Pessimism Removal)

Clock Path Example:

CLK
|
BUF1
|
BUF2
/
FF1 FF2

BUF1 and BUF2 are common to both clock paths.

Without CPPR:

Launch Common Path = 90 ps

Capture Common Path = 110 ps

Difference = 20 ps

Physically impossible because same buffer cannot have two delays simultaneously.

CPPR removes this pessimism.

Benefits:

- Better slack

- More realistic timing

# 18. Crosstalk

Occurs due to neighboring nets.

Aggressor Net

Victim Net

Switching aggressor affects victim.

Effects:

- Delay Increase

- Delay Reduction

- Noise Glitches

Signal Integrity STA includes these effects.

# 19. RC Parasitics

Routing introduces:  Resistance (R) and Capacitance (C).

These affect delay.

Delay approximately depends on: R × C

Source Files: SPEF and DSPF

Accurate extraction is critical.

# 20. Process Corners

Manufacturing variation creates different process conditions.

Common Corners: FF = Fast-Fast , TT = Typical-Typical and SS = Slow-Slow.

Generally: SS → Worst Setup and FF → Worst Hold

# 21. Voltage Impact

Higher Voltage: Faster Cells and Lower Delay

Lower Voltage: Slower Cells and Higher Delay

Example: 1.0V Delay = 100 ps , 0.8V Delay = 130 ps

# 22. Temperature Impact

Higher Temperature usually increases delay.

Example: 25°C Delay = 100 ps and  125°C Delay = 120 ps

Result: Higher Temperature → Slower Circuit

# 23. Library Timing Arcs

Example: NOR2 Gate

Inputs: A , B  Output: Y

Timing Arcs:   A → Y  ,  B → Y

Each arc has independent delay tables.

STA calculates timing based on the active arc.

# 24. MMMC (Multi-Mode Multi-Corner)

Checks timing across:

Multiple Modes:

- Functional

- Scan

- Test

Multiple Corners:

- FF

- TT

- SS

Goal: Single signoff covering all scenarios.

# 25. What Impacts Timing Most?

Major Timing Factors:

- Cell Delay

- Net Delay

- Clock Latency

- Clock Skew

- Clock Uncertainty

- Jitter

- OCV

- AOCV

- POCV

- Derates

- CPPR

- Crosstalk

- RC Parasitics

- Process Corner

- Voltage

- Temperature

- Setup Time

- Hold Time

- Library Characterization

- MMMC Constraints

# 26. Complete Setup Equation

Arrival Time (DAT) = Launch Clock Delay

- Clock-to-Q

- Logic Delay

- Net Delay

- OCV/AOCV/POCV Effects

- Crosstalk Effects

Required Time (DRT) = Clock Period

- Capture Clock Delay

- Setup Time

- Clock Uncertainty

Setup Slack = DRT - DAT

# 27. Complete Hold Equation

Arrival Time = Launch Clock Delay

- Clock-to-Q

- Logic Delay

- Net Delay

Required Time = Capture Clock Delay

- Hold Time

- Clock Uncertainty

Hold Slack  = Arrival Time - Required Time

# 28. STA Interview Summary

If asked:

"What factors affect timing in STA?"

Answer:

Clock latency, clock skew, clock uncertainty, jitter, setup and hold requirements, OCV/AOCV/POCV variations, timing derates, CPPR, process-voltage-temperature (PVT) corners, RC parasitics, crosstalk effects, library characterization, net delays, cell delays, and MMMC scenarios all impact timing calculations and final slack.