Timing report calculation: 



the arrival time

# STA reports the timing arc through which the analyzed signal propagates. Although the NOR gate has both A→Y and B→Y timing arcs, only the arc corresponding to the actual timing path is shown in the report.

Timing Report & STA Concepts – Quick Reference

## 1. Arrival Time Calculation

Arrival Time is calculated as:

Arrival Time = Previous Arrival Time + Current Arc Delay

Example:

Launch Clock Arrival = 3000 ps

DFF CK→Q Delay = 366 ps

Arrival at Q = 3000 + 366 = 3366 ps

Next Gate Delay (NOR2X1) = 168 ps

Arrival = 3366 + 168 = 3534 ps

Next Gate Delay (OR2X4) = 171 ps

Arrival = 3534 + 171 = 3705 ps

Thus, the arrival column is obtained by continuously adding delays along the path.

## 2. Timing Report Columns

### Arc

Timing relationship used by STA.

Examples:

CK→Q

A→Y

B→Y

### Fanout

Number of loads driven by the output pin.

Higher fanout generally increases load and delay.

### Load (Capacitance)

Total capacitance driven by the cell output.

Higher load increases delay.

### Transition (Slew)

Rise/Fall edge speed at the pin.

Large transition causes larger delays.

### Delay

Delay contributed by the current cell/net.

### Arrival

Cumulative delay from launch point up to the current point.

## 3. Relationship Between Columns

Fanout ↑
↓
Load ↑
↓
Delay ↑
↓
Arrival ↑

Transition ↑
↓
Delay ↑
↓
Arrival ↑

STA computes delay using Liberty (.lib) tables:

Cell Delay = f(Input Slew, Output Load)

## 4. NOR Gate Timing Arc

For a NOR2 gate:

A

\

NOR ---> Y

/

B

The library contains separate arcs:

A→Y

B→Y

A timing report may show:

NOR2X1 B→Y

This means the analyzed path propagated through pin B.

It does NOT mean pin A is ignored.

## 5. If A Arrives Later Than B

Example:

A arrival = 5 ns

B arrival = 3 ns

A→Y delay = 0.15 ns

B→Y delay = 0.20 ns

Output contributions:

A path = 5.15 ns

B path = 3.20 ns

Output Arrival = MAX(5.15, 3.20)

Output Arrival = 5.15 ns

For setup analysis, STA propagates the worst-case (latest) arrival.

## 6. Why Timing Report Shows Only One Arc

STA evaluates all relevant inputs internally.

The report displays only the arc that belongs to the reported timing path.

Example:

FF → B → NOR → Y

Report shows:

B→Y

Even though STA has already evaluated A→Y internally.

## 7. Setup Timing Equation

Setup Slack = Required Time − Arrival Time

Where:

Arrival Time = Launch Clock Arrival + Clock-to-Q Delay + Cell Delays + Net Delays
Required Time = Capture Clock Arrival + Clock Period − Setup Time − Uncertainty

If Slack > 0 → Timing Met

If Slack < 0 → Setup Violation

## 8. Common Delay Contributors

Large Fanout ->  Large Capacitance - > Poor Transition (High Slew)

Complex Logic Gates (MUX, AOI, OAI, NOR)

Long Nets -> Weak Drive Strength Cells

## 9. Interview Answers

Q: How is arrival time calculated?

A: Current Arrival = Previous Arrival + Current Arc Delay.

Q: Why does the report show B→Y and not A→Y?

A: Because the reported timing path propagates through pin B. The NOR gate contains both A→Y and B→Y timing arcs.

Q: If A arrives later than B, will timing be impacted?

A: Yes. STA evaluates all relevant inputs and propagates the worst-case arrival to the output.

Q: What determines cell delay?

A: Input slew and output load from Liberty timing tables.

# STA Fundamentals: DAT, DRT, Setup, Hold, Slack

## 1. DAT (Data Arrival Time)

DAT is the time at which data reaches the capture flip-flop D pin.

Formula:

DAT = Launch Clock Arrival + Clock-to-Q Delay + Data Path Delay

Where:

Data Path Delay = Cell Delays + Net Delays

Example:

Launch Clock Arrival = 3.0 ns

Clock-to-Q = 0.3 ns

Combinational Delay = 1.5 ns

DAT = 3.0 + 0.3 + 1.5

DAT = 4.8 ns

## 2. DRT (Data Required Time)

DRT is the latest time by which data must arrive at the capture flip-flop D pin.

Formula:

DRT = Capture Clock Arrival + Clock Period - Setup Time - Clock Uncertainty

Example:

Capture Clock Arrival = 3.1 ns

Clock Period = 5 ns

Setup Time = 0.1 ns

Uncertainty = 0.1 ns

DRT = 3.1 + 5 - 0.1 - 0.1 = 7.9

## 3. Setup Check

Setup check ensures data reaches the destination flip-flop before the next active clock edge.

Condition:

DAT < DRT

If data arrives too late, setup violation occurs.

Example:

DAT = 7.5 ns

DRT = 7.9 ns

Timing Met

Example:

DAT = 8.2 ns

DRT = 7.9 ns

Setup Violation

## 4. Setup Slack

Formula:

Setup Slack = DRT - DAT

Example:

DRT = 7.9 ns

DAT = 7.5 ns

Setup Slack = 7.9 - 7.5

Setup Slack = +0.4 ns

Positive Slack → Timing Met

Negative Slack → Setup Violation

## 5. Hold Check

Hold check ensures data does NOT reach the destination flip-flop too early after the same clock edge.

Condition:

DAT > Hold Requirement

## 6. Hold Required Time

For hold analysis:

Hold Required Time = Capture Clock Arrival + Hold Time + Clock Uncertainty

Example:

Capture Clock Arrival = 0.1 ns

Hold Time = 0.05 ns

Uncertainty = 0.05 ns

Hold Required Time = 0.2 ns

## 7. Hold Slack

Formula:

Hold Slack = DAT - Hold Required Time

Example: DAT = 0.4 ns

Hold Required Time = 0.2 ns

Hold Slack = 0.4 - 0.2

Hold Slack = +0.2 ns

Timing Met

Example:

DAT = 0.1 ns

Hold Required Time = 0.2 ns

Hold Slack = 0.1 - 0.2

Hold Slack = -0.1 ns

Hold Violation

## 8. Quick Comparison

Setup Analysis
Purpose:  Checks data is not late.
Formula: Setup Slack = DRT – DAT
Requirement: DAT < DRT
Violation: Negative Setup Slack

Hold Analysis
Purpose: Checks data is not early.
Formula: Hold Slack = DAT - Hold Required Time

Requirement: DAT > Hold Required Time

Violation: Negative Hold Slack

## 9. Easy Way to Remember

Setup: "Data must arrive BEFORE required time."

Setup Slack = Required - Arrival

Hold: "Data must remain stable AFTER clock edge."

Hold Slack = Arrival - Required

## 10. Interview Answer

DAT (Data Arrival Time): Actual time data reaches destination D pin.

DRT (Data Required Time): Latest allowable arrival time for setup analysis.

Setup Slack: DRT - DAT

Hold Slack: DAT - Hold Required Time

Positive Slack:Timing Met

Negative Slack:Timing Violation