# 1. What is Floor planning?

Floor planning is the process of deciding:

- Chip size (Core Area)

- Macro placement (SRAMs, Memories, IPs)

- IO placement

- Power structure planning

- Routing space reservation

- Utilization setting

### Goal : Create a layout structure that can achieve:

- Better timing

- Lower congestion

- Lower power

- Easier routing

- Better IR Drop performance

# 2. PD Flow Overview

RTL

↓

Synthesis

↓

Floorplanning

↓

Power Planning

↓

Placement

↓

CTS

↓

Routing

↓

STA & Signoff

↓

GDSII

### Previous Stage Input

Synthesis provides:  Netlist (.v) , SDC (.sdc) , MMMC views and Timing reports.

### Next Stage

Placement stage.

# 3. Inputs Required for Floorplan

## Logical Inputs

### Netlist : design.v

contains: Cells , Connections and Hierarchy

### Constraints : design.sdc
Contains: Clocks, Input delays, Output delays, False paths and Multicycle paths

## Physical Inputs

### LEF Files : tech.lef, stdcell.lef, and macro.lef.
Contain: Cell dimensions, Pin locations and Routing layers.

### Technology Files : tf, ict, tlu+ and captable.
Contain RC information.

Timing Libraries : .lib
Contain: Setup, Hold, Cell delays and Power data.

### MMMC File
Contains: Corners, Modes and RC corners

# 4. Outputs of Floorplanning

Typically:

design_floorplan.enc

design_floorplan.def

Contains:

- Core size

- Macro locations

- IO locations

- Blockages

- Power structure

Given to placement team/stage.

# 5. Main Activities Done by PD Engineer

## Activity 1: Decide Core Area

Formula: Core Area = Netlist Area ÷ Utilization
Example: Netlist Area = 1,000,000 µm²
Utilization = 70%
Core Area =1,000,000/0.7
=1,428,571 µm²

## Activity 2: Decide Aspect Ratio

Aspect Ratio: Height / Width
Examples: 1.0 → Square
0.5 → Wide
2.0 → Tall
Typical: 0.8 to 1.2

Activity 3: Macro Placement

Place:

- SRAM

- ROM

- Analog IP

- PLL

- DSP blocks

Rules:

- Near related logic

- Avoid center blockage

- Leave routing channels

Example:

Bad:

MEM1 MEM2 MEM3

MEM4 MEM5 MEM6

Huge congestion

Good:

MEM1     MEM2

Channel

MEM3     MEM4

## Activity 4: IO Placement

Place: Inputs, Outputs and Clock pins.
Example: Clock input near PLL

## Activity 5: Create Blockages
Prevent placement.

Types: Hard Blockage, Soft Blockage and Partial Blockage.
Used to reduce congestion.

Activity 6: Power Planning
Create: Power Ring

VDD

VSS

Create: Power Stripes

Purpose: Reduce IR Drop and Reduce EM violations

## Activity 7: Pin Assignment

For hierarchical blocks: Place pins strategically.

Bad pin placement causes: Congestion, Long routes and Timing issues.

# 6. Optimization Techniques Used During Floorplan

## Macro Clustering : Place communicating macros together.
Benefit: Short wire length and Better timing

Channel Spacing : Reserve routing space.
Benefit: Less congestion

Utilization Optimization : Typical: 65% - 75%

Too High: Congestion
Too Low: Larger chip area

## Macro Orientation : Rotate:  R0, R90, MX and MY

To reduce wirelength.

## Feed through Planning

Reserve path for signals crossing macros.

## Power Optimization

Increase power stripes.
Benefit: Lower IR Drop

# 7. Checks Done in Floorplan

## Floorplan Review

Check:  Core size, Aspect ratio and Macro placement.

## Congestion Check

Identify: Routing hotspots

## Timing Estimation

Check: Long paths

Power Check

Check:  IR Drop

## DRC Check

Check: Spacing and Overlap

# 8. Common Floorplan Commands (Innovus)

floorPlan : Create floorplan.

addRing : Create power ring.

addStripe : Create power stripes.

placeInstance : Place macros.

createRouteBlk : Create blockage.

reportCongestion : Check congestion.

# 9. Important Interview Questions

### Q1. What is floorplanning?

Floorplanning is deciding core area, aspect ratio, macro placement, IO placement and power planning before placement.

### Q2. What are floorplan inputs?

- Netlist, SDC, LEF, LIB, Technology files and MMMC

### Q3. What is utilization?

Cell Area/Core Area ×100

### Q4. What happens if utilization is 90%?

- Congestion increases, Timing degrades and Routing becomes difficult

### Q5. What is aspect ratio?

Height / Width

### Q6. Why are channels required between macros?

To provide routing resources.

### Q7. What is a hard blockage?

No cells can be placed.

### Q8. What is a soft blockage?

Placement discouraged but allowed.

### Q9. What is a power ring?

Metal ring around core carrying VDD and VSS.

### Q10. Why is floorplanning important?

About 70% of timing, congestion, power and routability success depends on floorplan quality.

# One-Line Interview Summary

Floorplanning is the stage where a PD engineer defines core size, utilization, aspect ratio, macro placement, IO placement, blockages, and power structure using inputs such as Netlist, SDC, LEF, LIB, MMMC, and technology files, and generates a DEF/database that is used by the Placement stage.

For interviews, after floorplanning, the next topic you should learn is Placement stage (global placement, legalization, congestion optimization, timing optimization, HFN buffering, scan chain reordering, etc.), because interviewers usually ask the complete flow from Floorplan → Placement → CTS.